PS14 — Autonomous Water-Sharing Dispute Mediation Agent

Topic-Wise Loopholes, Edge Cases & Solutions

# 1. Water Availability & Supply

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Sudden reduction in available water | Actual source water falls below the planned quantity. | Take fresh source measurement, invalidate stale schedule, recalculate allocation and notify affected farmers. |
| 2 | Sudden increase in available water | More water becomes available after the schedule is created. | Validate the increase and offer additional water only after checking fairness and safety constraints. |
| 3 | Wrong/manual water availability entry | Someone enters an incorrect quantity to influence allocation. | Prefer measured data; require timestamp/source for manual values; flag abnormal overrides and keep an audit record. |
| 4 | Bad source sensor reading | Sensor gives impossible, stuck or rapidly changing values. | Apply range and rate-of-change checks, sensor-health checks and cross-checks with other measurements. |
| 5 | Water arrives late | The schedule assumes water arrives exactly on time. | Use actual flow detection to activate/shift slots instead of relying only on the clock. |
| 6 | Water quality becomes unsuitable | Water quantity is sufficient but quality may make irrigation unsafe or unsuitable. | Add a quality-alert state and follow predefined authority rules for affected allocations. |

# 2. Farmer Requirement Manipulation

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Deliberately overstated demand | Farmer asks for much more water than required. | Compare demand with approved crop/area/season ranges and cap allocation at validated demand. |
| 2 | False urgency | Farmer claims an emergency to get priority. | Use crop stage, deadline and predefined evidence/rules; never give priority only because the message sounds urgent. |
| 3 | Requirement changed after allocation | A farmer changes quantity or time after the schedule is published. | Convert the change into new constraints and recalculate the affected schedule. |
| 4 | Repeated requirement changes | Frequent changes may be used to manipulate other farmers' schedules. | Track change history and trigger review or limit changes after repeated manipulation. |
| 5 | Cancellation after allocation | Farmer cancels a slot and the water may remain unused. | Release the unused capacity and offer it to the next eligible farmer according to the rules. |
| 6 | Allocated water not used | A farmer receives a slot but does not actually withdraw water. | Compare scheduled time with outlet-flow readings and reassign unused capacity where policy permits. |
| 7 | Unrealistic preferred time | Farmer demands a time already occupied or physically impossible. | Treat preference as a soft constraint and provide the nearest feasible alternative. |

# 3. Water Theft & Unauthorized Withdrawal

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Unauthorized withdrawal | Water disappears from the canal or a farmer takes more than authorized. | Compare sequential canal measurements and farmer outlet meters with the authorized schedule; flag unexplained loss. |
| 2 | Farmer takes more than allocated | Actual outlet quantity exceeds the approved quantity. | Compare outlet meter + gate position + schedule; flag excess withdrawal and apply predefined authority action. |
| 3 | Gate manipulation | A gate is opened differently from the authorized position. | Use gate-position measurement and compare commanded versus actual position. |
| 4 | Sensor tampering | A sensor is covered, moved, disconnected or manipulated. | Monitor device heartbeat/tamper status and cross-check with nearby measurements; mark suspicious data as untrusted. |
| 5 | Collusion between farmers | Multiple farmers may coordinate false claims or withdrawals. | Use independent measurement points and historical consistency checks; require human review before penalties. |
| 6 | Upstream over-withdrawal | An upstream farmer takes excess water and downstream farmers receive less. | Measure sequentially along the canal and localize the unexplained loss to a canal segment/outlet. |

# 4. Leakage, Seepage & Evaporation

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Canal leakage | Damaged infrastructure causes abnormal water loss. | Compare upstream/downstream measurements and identify the section with abnormal loss; create infrastructure-leak alert. |
| 2 | Seepage | Water gradually escapes through soil/canal walls and may look like theft. | Build a normal-loss baseline for each section and classify only the unexplained excess as an anomaly. |
| 3 | Evaporation | Open water naturally loses water, especially with heat, wind and long exposure. | Estimate expected physical loss using weather, exposed surface, time and historical patterns; do not treat expected evaporation as theft. |
| 4 | Heavy rainfall | Rain changes water level or supply unexpectedly. | Use rainfall and source-flow measurements to distinguish natural inflow from unauthorized withdrawal. |
| 5 | Flood/overflow | Water exceeds canal capacity and is lost through overflow. | Monitor water level thresholds and generate overflow alerts; temporarily adjust allocation if required. |
| 6 | Leak after farmer outlet | Farmer is allocated correctly but water is lost before reaching the intended field point. | Compare outlet and downstream measurements and investigate the physical segment. |

# 5. Under-Delivery & Fairness

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Farmer receives less than allocated | Example: 400 authorized but only 280 delivered. | Calculate shortfall = allocated − measured delivery; investigate upstream loss, gate issue or timing; apply compensation policy. |
| 2 | Repeated under-delivery | The same farmer repeatedly receives less water. | Track delivery history and satisfaction ratio; increase investigation priority and apply fairness correction. |
| 3 | Same farmer repeatedly gets priority | A scheduling algorithm may accidentally favor one farmer. | Include historical allocation, waiting time and need in fairness scoring. |
| 4 | Minimum needs exceed supply | All farmers' minimum requirements cannot be satisfied simultaneously. | Run feasibility check; clearly show the conflict and negotiate reductions/rescheduling instead of violating hard constraints. |
| 5 | Equal quantity is not fair | Equal distribution can ignore crop stage or critical need. | Use need-aware fairness and predefined crop/urgency rules. |
| 6 | Large versus small land area | Equal sharing may not match legitimate demand. | Use approved area/crop demand rules as constraints rather than arbitrary AI decisions. |

# 6. Scheduling & Canal Capacity

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Two farmers get the same slot | A single-capacity canal cannot serve both simultaneously. | Hard constraint prevents overlap and the system generates another feasible slot. |
| 2 | Continuous irrigation requirement | Splitting water into small slots may harm the crop. | Treat continuity as a relevant constraint/preference based on crop rules. |
| 3 | Canal maintenance | A canal section becomes unavailable after scheduling. | Mark it unavailable and automatically reroute/reschedule affected allocations. |
| 4 | Farmer unavailable at assigned time | Farmer cannot operate the outlet during the proposed slot. | Store availability windows and optimize around them where possible. |
| 5 | Water does not arrive at slot start | Clock says slot started but there is no flow. | Use actual flow as execution confirmation and shift the schedule. |
| 6 | Farmer misses slot | Allocated water may be wasted. | Detect zero/low outlet flow and release remaining water according to predefined rules. |

# 7. Negotiation & Objections

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Vague objection | Farmer says: 'This is unfair.' | AI converts the objection into a structured issue such as quantity, timing, priority or delivery problem. |
| 2 | Impossible demand | Farmer demands more than the system can physically provide. | Run feasibility check first and present mathematically feasible alternatives. |
| 3 | Accepts then changes mind | Agreement becomes unstable. | Allow controlled renegotiation before execution and preserve schedule versions. |
| 4 | Emotional manipulation | Persuasive language could make an AI favor one farmer. | Allocation must use structured constraints/rules, not emotional tone or writing quality. |
| 5 | Repeated objections to delay others | A stakeholder repeatedly rejects proposals without a new constraint. | Limit duplicate objections and escalate persistent non-resolution. |
| 6 | Same preferred slot demanded by two farmers | Both cannot receive the same physical resource at once. | Use priority/fairness rules, explain the trade-off and offer the closest alternative. |

# 8. AI / ML Loopholes

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | AI invents a rule | LLM may generate a rule that is not part of policy. | Binding rules must come only from the rule engine/policy store; AI only explains and negotiates. |
| 2 | AI misunderstands a request | Natural-language requests may be ambiguous. | Convert text to structured fields and ask for confirmation when confidence is low. |
| 3 | AI favors persuasive users | Better communication could accidentally receive better allocation. | Separate language understanding from allocation scoring. |
| 4 | Incorrect explanation | AI may give a confident explanation that does not match the actual decision. | Generate explanations from recorded constraints, measurements and schedule versions. |
| 5 | False anomaly alarm | ML may label normal water variation as theft. | Combine ML with thresholds, physical water balance, sensor health and human review. |
| 6 | Too little historical data | ML cannot reliably learn normal patterns for a new canal. | Start with rule/physics-based detection and add ML as sufficient historical data becomes available. |

# 9. IoT / Sensor / Hardware Edge Cases

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Sensor failure | Missing or stuck measurements distort the water balance. | Use health checks, missing-data status, redundancy for critical points and fallback logic. |
| 2 | Calibration drift | Sensor gradually becomes inaccurate. | Perform periodic calibration and compare against reference measurements. |
| 3 | Network failure | Measurements cannot reach the central system in real time. | Store readings locally, retry transmission and mark delayed data instead of treating it as zero. |
| 4 | Power failure | Sensor stops during an important allocation period. | Use backup power where practical and monitor device heartbeat. |
| 5 | Sensors disagree | Two nearby sensors show significantly different values. | Check timestamp alignment, calibration and sensor health before making any accusation. |
| 6 | Delayed reading | Old data is mistaken for current data. | Every measurement must have a timestamp and freshness status. |
| 7 | Level mistaken for volume | Water level alone does not always equal water quantity. | Use calibrated canal/tank geometry or appropriate flow measurement. |

# 10. Dynamic Changes / Real-Time Edge Cases

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Farmer adds a new request | New demand appears after allocation is already made. | Insert the request into the current state and recompute only what is affected while preserving existing commitments where possible. |
| 2 | Farmer withdraws request | A farmer no longer needs water. | Release the unused allocation and re-optimize the remaining capacity. |
| 3 | Emergency crop condition | A crop suddenly becomes time-critical. | Validate the emergency and recalculate priority using predefined emergency rules. |
| 4 | Canal blockage | Physical blockage reduces flow capacity. | Detect abnormal upstream/downstream difference, mark capacity reduced and reschedule affected farmers. |
| 5 | Sudden infrastructure repair | A section becomes unavailable during delivery. | Stop/flag the affected route, preserve actual delivery records and create a revised schedule. |
| 6 | Multiple simultaneous changes | Supply, demand and sensor readings all change together. | Process changes as one new system state, validate data first, then perform one controlled re-optimization. |

# 11. Audit, Accountability & Security

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Decision cannot be explained | Farmer asks why they received less water. | Store rules, inputs, measurements, score/fairness factors and schedule version used for the decision. |
| 2 | Past schedule is overwritten | It becomes impossible to prove what was originally agreed. | Keep immutable/versioned schedules and record every revision. |
| 3 | Manual override by authority | An authorized person changes an allocation. | Require reason, identity/role, timestamp and affected farmers; preserve previous version. |
| 4 | False complaint | Farmer claims they received less despite correct delivery. | Use outlet measurements and timestamped records as evidence. |
| 5 | Data manipulation | Someone edits historical readings or allocation records. | Use access control, audit logs and tamper-evident records. |
| 6 | Unresolved dispute | Farmers cannot reach an agreement. | Do not force a false agreement; preserve the latest proposal and escalate to the responsible authority. |

# 12. No-Feasible-Solution Cases

| No. | Loophole / Edge Case | How It Can Happen / Detection | Solution / System Response |
| --- | --- | --- | --- |
| 1 | Total minimum demand > available water | Example: A=400, B=350, C=300 minimums; supply=1000, total minimum=1050. | Declare the constraints infeasible, show the conflict, and negotiate reduced quantities/rescheduling/authorized alternatives. |
| 2 | All farmers demand the same time | Physical canal capacity cannot satisfy all simultaneously. | Keep capacity as a hard constraint and negotiate different time slots. |
| 3 | Demand exceeds canal capacity even with enough total water | Enough water exists overall but cannot be delivered fast enough. | Respect flow/canal capacity and split or reschedule delivery. |
| 4 | Emergency conflicts with an existing critical allocation | Two high-priority cases compete for the same limited water. | Apply predefined emergency and fairness rules; explain the trade-off and escalate if required. |

# 13. Water-Balance Detection — Key Formula

Expected downstream water = upstream measured water − authorized withdrawals − expected physical losses + relevant inflows

If measured downstream water is significantly lower than expected, the difference becomes an 'unexplained water loss'. Sequential measurement points can localize the suspicious canal section.

Example: 1000 units enter → A authorized 300 → B authorized 400 → expected physical loss 100 → expected remaining 200. If downstream measurement is 50, unexplained loss = 150 units. The system investigates the segment and outlet readings instead of automatically accusing a farmer.

# 14. Final System Response to Any Edge Case

| Step | Observe | Validate | Decide | Record / Escalate |
| --- | --- | --- | --- | --- |
| 1 | Farmer request, water, schedule, sensor data | Check timestamp, range, sensor health | Accept valid state changes | Log inputs |
| 2 | Detect shortage/loss/conflict | Cross-check measurements and rules | Classify issue | Create alert |
| 3 | Generate allocation | Check hard constraints and fairness | Select feasible schedule | Store schedule version |
| 4 | Receive objection | Interpret objection into structured constraints | Negotiate and re-optimize | Store objection/reason |
| 5 | Monitor actual delivery | Compare planned vs actual | Detect under/over-delivery | Record evidence |
| 6 | Unresolved/enforcement case | Check evidence quality | Escalate to authority | Preserve complete audit trail |

Important: The system should flag suspected theft or tampering based on evidence; it should not automatically declare a farmer guilty. Enforcement and penalties should follow predefined authority rules and, where required, human review.